Com_config.h

#ifndef _COM_CONFIG_H

#define _COM_CONFIG_H

#include "main.h"

// 连接状态

typedef enum

{

REMOTE_CONNECTED = 0,

REMOTE_DISCONNECTED,

}Remote_State;

// 飞行状态

typedef enum

{

IDLE = 0,

NORMAL,

FIX_HEIGHT,

FAIL,

}Flight_State;

// 油门解锁状态

typedef enum

{

FREE = 0,

MAX,

LEAVE_MAX,

MIN,

UNLOCK,

} Thr_state;

typedef struct

{

int16_t thr;

int16_t yaw;

int16_t pit;

int16_t rol;

uint8_t shutdown;   // 1: 关闭  0: 不关机

uint8_t fix_height; // 1. 切换定高和不定高 0: 不切换

} Remote_Data;

#endif // !_COM_CONFIG_H

App_receive_data.h

#ifndef __APP_RECEIVE_DATA__

#define __APP_RECEIVE_DATA__

#include "Int_SI24R1.h"

#include "Com_config.h"

// 定义帧头校验的值

#define FRAME_HEAD_CHECK_1 'c'

#define FRAME_HEAD_CHECK_2 's'

#define FRAME_HEAD_CHECK_3 'r'

// 最大重试次数

#define MAX_RETRY_TIMES 10

/**

* @brief 接收遥控器发送的遥控数据 => 解析为结构体

*

* @return uint8_t 0:校验通过 是正常的数据 1:没收到数据 或者 校验失败

*/

uint8_t App_receive_data(void);

/**

* @brief 处理连接状态的状态

*

* @param res 上一次接收数据的返回值

*/

void App_process_connect_state(uint8_t res);

/**

* @brief 处理飞机的飞行状态

*

*/

void App_process_flight_state(void);

#endif // __APP_RECEIVE_DATA__

App_receive_data.c

#include "App_receive_data.h"

extern Remote_Data remote_data;

uint8_t rx_buff[TX_PLOAD_WIDTH] = {0};

// 遥控连接状态

extern Remote_State remote_state;

// 飞行状态

extern Flight_State flight_state;

// 油门解锁状态值

Thr_state thr_state = FREE;

// MAX状态的进入时间

uint32_t max_enter_time = 0;

// MIN状态的进入时间

uint32_t min_enter_time = 0;

// 重试次数

uint8_t retry_count = 0;

/**

* @brief 接收遥控器发送的遥控数据 => 解析为结构体

*

* @return uint8_t 0:校验通过 是正常的数据 1:没收到数据 或者 校验失败

*/

uint8_t App_receive_data(void)

{

memset(rx_buff, 0, TX_PLOAD_WIDTH);

Int_SI24R1_RxPacket(rx_buff);

if (strlen((char *)rx_buff) == 0)

{

return 1;

}

// 1. 帧头校验

if (rx_buff[0] != FRAME_HEAD_CHECK_1 || rx_buff[1] != FRAME_HEAD_CHECK_2 || rx_buff[2] != FRAME_HEAD_CHECK_3)

{

return 1;

}

// 2. 帧尾校验

uint32_t sum = 0;

uint32_t sum_receive = 0;

for (uint8_t i = 0; i < 13; i++)

{

sum += rx_buff[i];

}

// 高位在前

sum_receive = rx_buff[13] << 24 | rx_buff[14] << 16 | rx_buff[15] << 8 | rx_buff[16];

if (sum != sum_receive)

{

return 1;

}

// 3. 保存数据

remote_data.thr = (rx_buff[3] << 8) | rx_buff[4];

remote_data.yaw = (rx_buff[5] << 8) | rx_buff[6];

remote_data.pit = (rx_buff[7] << 8) | rx_buff[8];

remote_data.rol = (rx_buff[9] << 8) | rx_buff[10];

remote_data.shutdown = rx_buff[11];

remote_data.fix_height = rx_buff[12];

debug_printf(":%d,%d,%d,%d,%d,%d\n", remote_data.thr, remote_data.yaw, remote_data.pit, remote_data.rol, remote_data.shutdown, remote_data.fix_height);

return 0;

}

/**

* @brief 处理连接状态的状态

*

* @param res 上一次接收数据的返回值

*/

void App_process_connect_state(uint8_t res)

{

if (res == 0)

{

// 接收数据成功一次 即为连接成功

// 此处使用的全局变量 只有当前一个地方会修改 LED灯控任务当中是读取使用

remote_state = REMOTE_CONNECTED;

retry_count = 0;

}

else if (res == 1)

{

// 接收数据失败 即为

retry_count++;

if (retry_count >= MAX_RETRY_TIMES)

{

remote_state = REMOTE_DISCONNECTED;

retry_count = 0;

}

}

}

/**

* @brief 处理解锁逻辑

*

* @return uint8_t 0: 解锁成功 1: 解锁失败

*/

static uint8_t App_process_unlock(void)

{

// 1. 考虑安全问题 =>  解锁完成的最终状态应该是油门为0

switch (thr_state)

{

case FREE:

if (remote_data.thr >= 900)

{

// 2. 进入max状态

thr_state = MAX;

// freeRTOS操作系统中以ms为单位计数的时间

max_enter_time = xTaskGetTickCount();

}

break;

case MAX:

// 3. 持续的时间应该是离开的时间减去进入的时间

if (remote_data.thr < 900)

{

if (xTaskGetTickCount() - max_enter_time >= 1000)

{

// 4. 油门保持最高状态超过1s => 进入leave_max状态

thr_state = LEAVE_MAX;

}

else

{

// 5. 油门保持最高状态时间小于1s => 退回到free 重新解锁

thr_state = FREE;

}

}

break;

case LEAVE_MAX:

if (remote_data.thr <= 100)

{

// 6. 油门回到0  进入min状态

thr_state = MIN;

min_enter_time = xTaskGetTickCount();

}

break;

case MIN:

// 7. 每次判断当前已经保持了多久

if (xTaskGetTickCount() - min_enter_time <= 1000)

{

// 还不够1s

if (remote_data.thr > 100)

{

thr_state = FREE;

}

}

else

{

// 已经保持够1s => 解锁完成

thr_state = UNLOCK;

}

break;

case UNLOCK:

/* code */

break;

default:

break;

}

if (thr_state == UNLOCK)

{

return 0;

}

return 1;

}

/**

* @brief 处理飞机的飞行状态

*

*/

void App_process_flight_state(void)

{

// 使用状态机逻辑实现

// 1. 轮询调用判断当前所处的状态

switch (flight_state)

{

case IDLE:

// 2. 只需要编写指向其他状态的代码即可

if (App_process_unlock() == 0)

{

flight_state = NORMAL;

// 每一次解锁成功  需要将解锁状态重置

thr_state = FREE;

}

break;

case NORMAL:

// 3. 判断进入定高

if (remote_data.fix_height == 1)

{

flight_state = FIX_HEIGHT;

remote_data.fix_height = 0;

}

// 4. 判断进入故障失联状态

if (remote_state == REMOTE_DISCONNECTED)

{

flight_state = FAIL;

}

break;

case FIX_HEIGHT:

// 5. 取消定高

if (remote_data.fix_height == 1)

{

flight_state = NORMAL;

remote_data.fix_height = 0;

}

// 6. 判断故障

if (remote_state == REMOTE_DISCONNECTED)

{

flight_state = FAIL;

}

break;

case FAIL:

// 7.处理失联故障  缓慢停止电机

// TODO

vTaskDelay(1);

flight_state = IDLE;

break;

default:

break;

}

}

App_freeRTOS_Task.h

#ifndef __APP_FREERTOS_TASK__

#define __APP_FREERTOS_TASK__

#include "FreeRTOS.h"

#include "task.h"

#include "Com_debug.h"

#include "Com_config.h"

#include "Int_IP5305T.h"

#include "Int_motor.h"

#include "Int_led.h"

#include "Int_SI24R1.h"

#include "App_receive_data.h"

/**

* @brief 启动freeRTOS操作系统

*

*/

void App_freeRTOS_start(void);

#endif // __APP_FREERTOS_TASK__

App_freeRTOS_Task.c

#include "App_freeRTOS_Task.h"

// 表示当前连接状态

Remote_State remote_state = REMOTE_DISCONNECTED;

// 表示当前的飞行状态

Flight_State flight_state = IDLE;

// 扩展获取接收的遥控数据

Remote_Data remote_data = {0};

// 通讯任务

void com_task(void *args);

#define COM_TASK_STACK_SIZE 128

#define COM_TASK_PRIORITY 2

TaskHandle_t com_task_handle;

// 任务周期

#define COM_TASK_PERIOD 6

/**

* @brief 启动freeRTOS操作系统

*/

void App_freeRTOS_start(void)

{

// 创建通讯任务

xTaskCreate(com_task, "com_task", COM_TASK_STACK_SIZE, NULL, COM_TASK_PRIORITY, &com_task_handle);

// 启动调度器

vTaskStartScheduler();

}

void com_task(void *args)

{

// 获取当前的基准时间

TickType_t xLastWakeTime = xTaskGetTickCount();

while (1)

{

// 1. 接收数据

uint8_t res = App_receive_data();

// 2. 根据接收数据的返回值 处理当前飞机的连接状态

App_process_connect_state(res);

// 3. 处理关机命令

if (remote_data.shutdown == 1)

{

// 使用Int_IP5305T_shutdown 关机  功能可以实现  但是项目结构比较奇怪

// Int_IP5305T_shutdown();

// 使用freeRTOS直接任务通知 => 通知电源任务 => 执行关机

xTaskNotifyGive(power_task_handle);

}

// 4. 处理飞机的飞行状态

App_process_flight_state();

// 6ms执行一次 接收数据的时间间隔应该等于发送数据的时间间隔

vTaskDelayUntil(&xLastWakeTime, COM_TASK_PERIOD);

}

}