Com_config.h

    - #ifndef _COM_CONFIG_H

    - #define _COM_CONFIG_H

    - #include "main.h"

    - // 连接状态

    - typedef enum

    - {

    - REMOTE_CONNECTED = 0,

    - REMOTE_DISCONNECTED,

    - }Remote_State;

    - // 飞行状态

    - typedef enum

    - {

    - IDLE = 0,

    - NORMAL,

    - FIX_HEIGHT,

    - FAIL,

    - }Flight_State;

    - // 油门解锁状态

    - typedef enum

    - {

    - FREE = 0,

    - MAX,

    - LEAVE_MAX,

    - MIN,

    - UNLOCK,

    - } Thr_state;

    - typedef struct

    - {

    - int16_t thr;

    - int16_t yaw;

    - int16_t pit;

    - int16_t rol;

    - uint8_t shutdown;   // 1: 关闭  0: 不关机

    - uint8_t fix_height; // 1. 切换定高和不定高 0: 不切换

    - } Remote_Data;

    - #endif // !_COM_CONFIG_H

App_receive_data.h

    - #ifndef __APP_RECEIVE_DATA__

    - #define __APP_RECEIVE_DATA__

    - #include "Int_SI24R1.h"

    - #include "Com_config.h"

    - // 定义帧头校验的值

    - #define FRAME_HEAD_CHECK_1 'c'

    - #define FRAME_HEAD_CHECK_2 's'

    - #define FRAME_HEAD_CHECK_3 'r'

    - // 最大重试次数

    - #define MAX_RETRY_TIMES 10

    - /**

    - * @brief 接收遥控器发送的遥控数据 => 解析为结构体

    - *

    - * @return uint8_t 0:校验通过 是正常的数据 1:没收到数据 或者 校验失败

    - */

    - uint8_t App_receive_data(void);

    - /**

    - * @brief 处理连接状态的状态

    - *

    - * @param res 上一次接收数据的返回值

    - */

    - void App_process_connect_state(uint8_t res);

    - /**

    - * @brief 处理飞机的飞行状态

    - *

    - */

    - void App_process_flight_state(void);

    - #endif // __APP_RECEIVE_DATA__

App_receive_data.c

    - #include "App_receive_data.h"

    - extern Remote_Data remote_data;

    - uint8_t rx_buff[TX_PLOAD_WIDTH] = {0};

    - // 遥控连接状态

    - extern Remote_State remote_state;

    - // 飞行状态

    - extern Flight_State flight_state;

    - // 油门解锁状态值

    - Thr_state thr_state = FREE;

    - // MAX状态的进入时间

    - uint32_t max_enter_time = 0;

    - // MIN状态的进入时间

    - uint32_t min_enter_time = 0;

    - // 重试次数

    - uint8_t retry_count = 0;

    - /**

    - * @brief 接收遥控器发送的遥控数据 => 解析为结构体

    - *

    - * @return uint8_t 0:校验通过 是正常的数据 1:没收到数据 或者 校验失败

    - */

    - uint8_t App_receive_data(void)

    - {

    - memset(rx_buff, 0, TX_PLOAD_WIDTH);

    - Int_SI24R1_RxPacket(rx_buff);

    - if (strlen((char *)rx_buff) == 0)

    - {

    - return 1;

    - }

    - // 1. 帧头校验

    - if (rx_buff[0] != FRAME_HEAD_CHECK_1 || rx_buff[1] != FRAME_HEAD_CHECK_2 || rx_buff[2] != FRAME_HEAD_CHECK_3)

    - {

    - return 1;

    - }

    - // 2. 帧尾校验

    - uint32_t sum = 0;

    - uint32_t sum_receive = 0;

    - for (uint8_t i = 0; i < 13; i++)

    - {

    - sum += rx_buff[i];

    - }

    - // 高位在前

    - sum_receive = rx_buff[13] << 24 | rx_buff[14] << 16 | rx_buff[15] << 8 | rx_buff[16];

    - if (sum != sum_receive)

    - {

    - return 1;

    - }

    - // 3. 保存数据

    - remote_data.thr = (rx_buff[3] << 8) | rx_buff[4];

    - remote_data.yaw = (rx_buff[5] << 8) | rx_buff[6];

    - remote_data.pit = (rx_buff[7] << 8) | rx_buff[8];

    - remote_data.rol = (rx_buff[9] << 8) | rx_buff[10];

    - remote_data.shutdown = rx_buff[11];

    - remote_data.fix_height = rx_buff[12];

    - debug_printf(":%d,%d,%d,%d,%d,%d\n", remote_data.thr, remote_data.yaw, remote_data.pit, remote_data.rol, remote_data.shutdown, remote_data.fix_height);

    - return 0;

    - }

    - /**

    - * @brief 处理连接状态的状态

    - *

    - * @param res 上一次接收数据的返回值

    - */

    - void App_process_connect_state(uint8_t res)

    - {

    - if (res == 0)

    - {

    - // 接收数据成功一次 即为连接成功

    - // 此处使用的全局变量 只有当前一个地方会修改 LED灯控任务当中是读取使用

    - remote_state = REMOTE_CONNECTED;

    - retry_count = 0;

    - }

    - else if (res == 1)

    - {

    - // 接收数据失败 即为

    - retry_count++;

    - if (retry_count >= MAX_RETRY_TIMES)

    - {

    - remote_state = REMOTE_DISCONNECTED;

    - retry_count = 0;

    - }

    - }

    - }

    - /**

    - * @brief 处理解锁逻辑

    - *

    - * @return uint8_t 0: 解锁成功 1: 解锁失败

    - */

    - static uint8_t App_process_unlock(void)

    - {

    - // 1. 考虑安全问题 =>  解锁完成的最终状态应该是油门为0

    - switch (thr_state)

    - {

    - case FREE:

    - if (remote_data.thr >= 900)

    - {

    - // 2. 进入max状态

    - thr_state = MAX;

    - // freeRTOS操作系统中以ms为单位计数的时间

    - max_enter_time = xTaskGetTickCount();

    - }

    - break;

    - case MAX:

    - // 3. 持续的时间应该是离开的时间减去进入的时间

    - if (remote_data.thr < 900)

    - {

    - if (xTaskGetTickCount() - max_enter_time >= 1000)

    - {

    - // 4. 油门保持最高状态超过1s => 进入leave_max状态

    - thr_state = LEAVE_MAX;

    - }

    - else

    - {

    - // 5. 油门保持最高状态时间小于1s => 退回到free 重新解锁

    - thr_state = FREE;

    - }

    - }

    - break;

    - case LEAVE_MAX:

    - if (remote_data.thr <= 100)

    - {

    - // 6. 油门回到0  进入min状态

    - thr_state = MIN;

    - min_enter_time = xTaskGetTickCount();

    - }

    - break;

    - case MIN:

    - // 7. 每次判断当前已经保持了多久

    - if (xTaskGetTickCount() - min_enter_time <= 1000)

    - {

    - // 还不够1s

    - if (remote_data.thr > 100)

    - {

    - thr_state = FREE;

    - }

    - }

    - else

    - {

    - // 已经保持够1s => 解锁完成

    - thr_state = UNLOCK;

    - }

    - break;

    - case UNLOCK:

    - /* code */

    - break;

    - default:

    - break;

    - }

    - if (thr_state == UNLOCK)

    - {

    - return 0;

    - }

    - return 1;

    - }

    - /**

    - * @brief 处理飞机的飞行状态

    - *

    - */

    - void App_process_flight_state(void)

    - {

    - // 使用状态机逻辑实现

    - // 1. 轮询调用判断当前所处的状态

    - switch (flight_state)

    - {

    - case IDLE:

    - // 2. 只需要编写指向其他状态的代码即可

    - if (App_process_unlock() == 0)

    - {

    - flight_state = NORMAL;

    - // 每一次解锁成功  需要将解锁状态重置

    - thr_state = FREE;

    - }

    - break;

    - case NORMAL:

    - // 3. 判断进入定高

    - if (remote_data.fix_height == 1)

    - {

    - flight_state = FIX_HEIGHT;

    - remote_data.fix_height = 0;

    - }

    - // 4. 判断进入故障失联状态

    - if (remote_state == REMOTE_DISCONNECTED)

    - {

    - flight_state = FAIL;

    - }

    - break;

    - case FIX_HEIGHT:

    - // 5. 取消定高

    - if (remote_data.fix_height == 1)

    - {

    - flight_state = NORMAL;

    - remote_data.fix_height = 0;

    - }

    - // 6. 判断故障

    - if (remote_state == REMOTE_DISCONNECTED)

    - {

    - flight_state = FAIL;

    - }

    - break;

    - case FAIL:

    - // 7.处理失联故障  缓慢停止电机

    - // TODO

    - vTaskDelay(1);

    - flight_state = IDLE;

    - break;

    - default:

    - break;

    - }

    - }

App_freeRTOS_Task.h

    - #ifndef __APP_FREERTOS_TASK__

    - #define __APP_FREERTOS_TASK__

    - #include "FreeRTOS.h"

    - #include "task.h"

    - #include "Com_debug.h"

    - #include "Com_config.h"

    - #include "Int_IP5305T.h"

    - #include "Int_motor.h"

    - #include "Int_led.h"

    - #include "Int_SI24R1.h"

    - #include "App_receive_data.h"

    - /**

    - * @brief 启动freeRTOS操作系统

    - *

    - */

    - void App_freeRTOS_start(void);

    - #endif // __APP_FREERTOS_TASK__

App_freeRTOS_Task.c

    - #include "App_freeRTOS_Task.h"

    - // 表示当前连接状态

    - Remote_State remote_state = REMOTE_DISCONNECTED;

    - // 表示当前的飞行状态

    - Flight_State flight_state = IDLE;

    - // 扩展获取接收的遥控数据

    - Remote_Data remote_data = {0};

    - // 通讯任务

    - void com_task(void *args);

    - #define COM_TASK_STACK_SIZE 128

    - #define COM_TASK_PRIORITY 2

    - TaskHandle_t com_task_handle;

    - // 任务周期

    - #define COM_TASK_PERIOD 6

    - /**

    - * @brief 启动freeRTOS操作系统

    - */

    - void App_freeRTOS_start(void)

    - {

    - // 创建通讯任务

    - xTaskCreate(com_task, "com_task", COM_TASK_STACK_SIZE, NULL, COM_TASK_PRIORITY, &com_task_handle);

    - // 启动调度器

    - vTaskStartScheduler();

    - }

    - void com_task(void *args)

    - {

    - // 获取当前的基准时间

    - TickType_t xLastWakeTime = xTaskGetTickCount();

    - while (1)

    - {

    - // 1. 接收数据

    - uint8_t res = App_receive_data();

    - // 2. 根据接收数据的返回值 处理当前飞机的连接状态

    - App_process_connect_state(res);

    - // 3. 处理关机命令

    - if (remote_data.shutdown == 1)

    - {

    - // 使用Int_IP5305T_shutdown 关机  功能可以实现  但是项目结构比较奇怪

    - // Int_IP5305T_shutdown();

    - // 使用freeRTOS直接任务通知 => 通知电源任务 => 执行关机

    - xTaskNotifyGive(power_task_handle);

    - }

    - // 4. 处理飞机的飞行状态

    - App_process_flight_state();

    - // 6ms执行一次 接收数据的时间间隔应该等于发送数据的时间间隔

    - vTaskDelayUntil(&xLastWakeTime, COM_TASK_PERIOD);

    - }

    - }