Com_config.h

#ifndef _COM_CONFIG_H

#define _COM_CONFIG_H

#include "main.h"

// 连接状态

typedef enum

{

REMOTE_CONNECTED = 0,

REMOTE_DISCONNECTED,

}Remote_State;

// 飞行状态

typedef enum

{

IDLE = 0,

NORMAL,

FIX_HEIGHT,

FAIL,

}Flight_State;

#endif // !_COM_CONFIG_H

Int_led.h

#ifndef __INT_LED__

#define __INT_LED__

#include "main.h"

typedef struct

{

GPIO_TypeDef *port;

uint16_t pin;

} LED_Struct;

/**

* @brief 打开LED灯

* @param led

*/

void Int_led_turn_on(LED_Struct *led);

/**

* @brief 关闭LED灯

* @param led

*/

void Int_led_turn_off(LED_Struct *led);

/**

* @brief 翻转LED灯

* @param led

*/

void Int_led_toggle(LED_Struct *led);

#endif // __INT_LED__

Int_led.c

#include "Int_led.h"

/**

* @brief 打开LED灯

* @param led

*/

void Int_led_turn_on(LED_Struct *led)

{

// 直接修改引脚电平为低电平  开灯

HAL_GPIO_WritePin(led->port, led->pin, GPIO_PIN_RESET);

}

/**

* @brief 关闭LED灯

* @param led

*/

void Int_led_turn_off(LED_Struct *led)

{

// 直接修改引脚电平为高电平  关灯

HAL_GPIO_WritePin(led->port, led->pin, GPIO_PIN_SET);

}

/**

* @brief 翻转LED灯

* @param led

*/

void Int_led_toggle(LED_Struct *led)

{

HAL_GPIO_TogglePin(led->port, led->pin);

}

App_freeRTOS_Task.c

#include "App_freeRTOS_Task.h"

// LED结构体

LED_Struct left_top_led = {.port = LED1_GPIO_Port, .pin = LED1_Pin};

LED_Struct right_top_led = {.port = LED2_GPIO_Port, .pin = LED2_Pin};

LED_Struct right_bottom_led = {.port = LED3_GPIO_Port, .pin = LED3_Pin};

LED_Struct left_bottom_led = {.port = LED4_GPIO_Port, .pin = LED4_Pin};

// 表示当前连接状态

Remote_State remote_state = REMOTE_DISCONNECTED;

// 表示当前的飞行状态

Flight_State flight_state = NORMAL;

// LED任务

void led_task(void *args);

#define LED_TASK_STACK_SIZE 128

#define LED_TASK_PRIORITY 1

TaskHandle_t led_task_handle;

#define LED_TASK_PERIOD 100

/**

* @brief 启动freeRTOS操作系统

*/

void App_freeRTOS_start(void)

{

// 创建LED灯任务

xTaskCreate(led_task, "led_task", LED_TASK_STACK_SIZE, NULL, LED_TASK_PRIORITY, &led_task_handle);

// 启动调度器

vTaskStartScheduler();

}

void led_task(void *args)

{

// 获取当前的基准时间

TickType_t xLastWakeTime = xTaskGetTickCount();

uint8_t count = 0;

while (1)

{

count++;

// 前两个灯表示连接状态

// 1. 判断当前连接状态

if (remote_state == REMOTE_CONNECTED)

{

// 点亮前两个灯

Int_led_turn_on(&left_top_led);

Int_led_turn_on(&right_top_led);

}

else if (remote_state == REMOTE_DISCONNECTED)

{

// 关掉前两个灯

Int_led_turn_off(&left_top_led);

Int_led_turn_off(&right_top_led);

}

// 后两个灯表示飞行状态

// 2. 判断当前飞行状态

if (flight_state == IDLE)

{

// 灯慢闪烁 => 500ms亮 500ms灭

if (count % 5 == 0)

{

// 循环5次  一次是100ms  5次等于500ms

Int_led_toggle(&left_bottom_led);

Int_led_toggle(&right_bottom_led);

}

}

else if (flight_state == NORMAL)

{

// 灯快闪  =>  200ms亮 200ms灭

if (count % 2 == 0)

{

// 循环2次  一次是100ms  2次等于200ms

Int_led_toggle(&left_bottom_led);

Int_led_toggle(&right_bottom_led);

}

}

else if (flight_state == FIX_HEIGHT)

{

// 后两个灯常量

Int_led_turn_on(&left_bottom_led);

Int_led_turn_on(&right_bottom_led);

}

else if (flight_state == FAIL)

{

// 后两个灯灭

Int_led_turn_off(&left_bottom_led);

Int_led_turn_off(&right_bottom_led);

}

// 将count计数重置

if (count == 10)

{

count = 0;

}

vTaskDelayUntil(&xLastWakeTime, LED_TASK_PERIOD);

}

}